BRP

User Manual

LF-BRP-UM-EN

v1.2

NOVEMBER 2025

www.liftena.com

DOCUMENT REVISION HISTORY

| İS NAME İM | TITLE | TITLE | DATE |
| --- | --- | --- | --- |
| Prepared by |  |  |  |
| M. Uygar AKGÜL | General Manager |  | 18.11.2025 |
| Checked by |  |  |  |
| M. Uygar AKGÜL | General Manager |  | 18.11.2025 |
| Approved by |  |  |  |
| M. Uygar AKGÜL | General Manager |  | 18.11.2025 |

DOKÜMAN YAYIN ÇİZELGESİ

| Revision No | Date | Section/Page Modified | Reason |
| --- | --- | --- | --- |
| 1.1 | SEP 2025 | Entire Document | First Release |
| 1.2 | NOW 2025 | Entire Document |  |

INDEX

1	GENERAL FEATURES	5

2	SYSTEM OVERVIEW	6

3	TECHNICAL SPECIFICATIONS	7

3.1	Input Supply	7

3.2	Output Options	7

4	TERMINOLOGY	8

5	DİMENSİONS (in mm)	9

6	NOTES	9

7	APPENDICES	9

# GENERAL FEATURES

The BRP is a coil voltage regulation module designed with flexible options to prevent brake/regulator coils from being continuously exposed to high current

# SYSTEM OVERVIEW

Şekil 1 Genel Görünüm

# TECHNICAL SPECIFICATIONS

# Input Supply

| Özellik | Açıklama |
| --- | --- |
| Supply Voltage | 220VAC |

Tablo 1 Input Supply

# Output Options

| Anahtar 1 | Anahtar 2 | Açıklama |
| --- | --- | --- |
| 220V | %100 | Output starts and continues at 220V |
| 220V | %75 | Output starts at 220V, drops to 170V after 3 seconds |
| 220V | %50 | Output starts at 220V, drops to 110V after 3 seconds |
| 220V | %25 | Output starts at 220V, drops to 55V after 3 seconds |
| 110V | %100 | Output starts and continues at 110V |
| 110V | %75 | Output starts at 110V, drops to 75V after 3 seconds |
| 110V | %50 | Output starts at 110V, drops to 55V after 3 seconds |
| 110V | %25 | Output starts at 110V, drops to 35V after 3 seconds |

Tablo 2 Output Options

# TERMINOLOGY

| Rumuz | Açıklama |
| --- | --- |
| AC | 220 VAC Input |
| AC | 220 VAC Input |
| - | Output Negative Terminal |
| + | Output Positive Terminal |

Tablo 3 Terminal Names

# DİMENSİONS (in mm)

# NOTES

None

# APPENDICES

None.